2. TEXTUAL DOCUMENTATION ________________________________________________

## 2.1.  Technical description – bridge structure

### General project data

| EMPLOYER: | European Investment Bank |
| --- | --- |
| BENEFICIARY: | Željeznička infrastruktura Crne Gore AD Podgorica |
| STRUCTURE: | BRIDGE no. 38, at km 330+649.90 |
| PROJECT: | MAIN DESIGN FOR REHABILITATION OF BRIDGE at km.330+649.90 |
| SECTION: | Vrbnica - Bar |
| CHAINAGE: | 330+649.90 |
| PROJECT STAGE: | Main design |
| NAME AND DESIGNATION OF PROJECT SECTION: | VOLUME II – STRUCTURAL DESIGN Bridge no.38, at km 330+649.90 |
| PROJECT ORGANIZATION RESPONSIBLE FOR DRAFTING BRIDGE STRUCTURAL DESIGN: | Cestra d.o.o. Beograd Makenzijeva 57, Beograd 11000 |

### Description of route location and alignment

- The bridge is located at chainage km 330+649.90 on the Vrbnica-Bar railway line.  The track is straight, and the level line of the bridge is lowered 1‰ towards Gostun. Bridge is a continuous beam structure with spans 10+15+10m. The bridge extends over a dry ravine, and the terrain is a steep cut. The track structure is that of CWR.

### Attachment 1 – Bridge layout plan from the ŽICG archives

### Attachment 2 – Technical Report from the original design

The main structural design was done by the Association of Yugoslav Railways, Designing Institute,  Belgrade, 1966.

The main design dates back to 1966.

### Schedule 3 – Assessment of bridge condition with proposed measures (Bridge inspection report 2015)

- Taken from „The report on the state of the bridge: the bridge at km 330+649.90 the railway line Vrbnica – Bar” from 2014.

- General observation about the state of the bridge:

By measuring the dimensions of structural elements and by comparison with layout sketches from the “Report about the current state of bridges on the Vrbnica-Bar railway line”, the following is concluded:

- all dimensions of the bridge are in compliance with the dimensions from the "Report about the current state of bridges on the Vrbnica-Bar railway line",

- after removing the track ballast and measuring of all necessary dimensions,  thickness of the RC slab was determined indirectly and it was found to be 23 cm (in the middle of the slab with a protective coating and waterproofing).

- the bridge dimensions comply with the dimensions from the "Report about the current state of bridges on the Vrbnica-Bar railway line",

- after removing the track ballast and measuring of all necessary dimensions,  thickness of the RC slab was determined indirectly and it was found to be 30 cm (in the middle of the slab with a protective coating and waterproofing and assuming sided cross inclination).

Railway track on the bridge is continuous welded rail (CWR) and in a good condition. After removing the track ballast in the sleeper zone, thickness of the track ballast below the lower edge of sleeper was found to be 20 cm (in the axis of railway track).

Damages that could reduce the capacity and stability of the bridge structure were not found during the detailed visual inspection of the bridge.

The observed damages and deficiencies of the structure which negatively affect the durability and functionality of the structure are the following:

- mechanical concrete damage in the main girder lower zone at places of measuring tape incorporation that has not been rehabilitated, the reinforcement is exposed and corroded (in spans S1-S2 and S2-S3),

- lack of waterproof expansion joints on sidewalk cantilevers results in increased water drain in zones of the construction dilatation,

- lack of drips on abutment S4 right wing wall cantilever, which leads to uncontrolled pouring under the cantilever,

- inadequate length of gullies, they are too short, which may causes wetting of the span structure,

- inadequate drainage method of the channels for installations –holes without gullies due to which profuse pouring of cantilevers concrete and lower zone of RC slab.

- concrete degradation of cantilevers in zone of gullies with visible and corroded reinforcement,

- concrete degradation of upper zone of sidewalks fascia with visible and corroded reinforcement,

- calcification of the entire lower zone of the cantilever for bearing the column for the electric power, with concrete degradation and visible and corroded reinforcement. On the cantilever there is no column – cantilever is out of function.

## Proposed measures from the aspect of the bridge structure durability and functionality

- concrete rehabilitation of the main girders lower zone at places of measuring tape incorporation,

- extension of waterproofing expansion joints on the part of cantilevers,

- creating drip edges on the abutment S4 right wing wall cantilever,

- gullies shall be extended under the level of lower structure edge (DIK) and perform incorporation of lacking gullies covers,

- to incorporate gullies into already existing holes for drainage of channels for installations,

- concrete rehabilitation of cantilevers in zone of gullies,

- concrete rehabilitation of upper zone of sidewalks fascia

For these works it is necessary to make the technical documentation which must include a technical description of works, the technology of the works, estimated bill of quantity, and the necessary technical details.

### Damage to the bridge and possible causes

A detailed summary of damage with possible causes was given in the previous project stage:

˝Report on bridge condition: bridge at km 330+649.90 on Vrbnica – Bar railway line˝ from 2015, which was prepared by Pro-Inženjering d.o.o. in Belgrade.

A graphic representation of damage observed on bridge inspection is given in graphic documents of the design file in the drawing – Photograph of damage.

Based on the “Report of bridge condition: bridge at km 330+649.90 on Vrbnica – Bar railway line˝ from 2015 the scope of damage was analysed alongside the methods for its rehabilitation. Scope of works and investment value were based on an inspection performed in 2015.

### Description of bridge structure – current condition

The bridge structure is a system of continuous beams with the spans 10+15+10m situated in the vicinity of Mojkovac. It extends over a dry valley on an inclined slope. The track on the bridge is straight and was produced as a CWR. The bridge is also straight. The total width of the bridge is 6.30m, with 95 cm wide footpaths on each side of the bridge.

The bridge structure comprises two reinforced concrete beams with a T-section and the axial distance between them is 2.80m. The total height of girders is 1.60m, and the slab is 25cm thick in the middle. The bridge deck is haunched on the underside and designed in a dual slope of 2%. Cantilevers of footpaths are 1.50m and are haunched with maximum thickness of 30cm. Longitudinal girder webs are 50 cm wide on a plane, whereas on intermediate supports they are haunched up to the width of 60cm. Girders are connected with cross members designed on a grid of 5m. Intermediate cross girders are 30cm thick, whereas the end and intermediate supports are 40cm thick. As far as statics is concerned, the continuous beam is supported on the movable supports on the pier S1, S2 and S3, while on the pier S4 there is a fixed support.

Girders, bridge deck, cross girders and pendulums were made using the MB 300 concrete grade, and piers were made using the MB 160 concrete. Bearing beams were made of MB 220 concrete, MB 110 was used for pier foundations. Rebar was made of Č-37 steel grade.

Piers have variable height cross-sections. Piers S2 and S3 measure b×d=4.0×1.20(0.80)m. Abutments basis measure b×d=3.0×4.80m – pier S1 and b×d=2.70×5.70m – pier S4. Both piers and abutments were to be founded on a solid compact rock mass. Data on the thickness of talus obtained during the geological investigations brought about a change in the level of foundation.

Structure layout

Longitudinal section

Structure base

##### Current condition:

The track on the bridge is straight and was produced as a CWR. The track axis coincides with the axis of the construction. The track is a closed one with a crushed stone ballast on the bridge deck. Damp-proofing was placed on the upper deck with a concrete cover of 5cm.

At the ends of the bridge deck, there are footpaths 95cm wide. There are cable ducts on footpaths covered with prefabricated RC covers that form the footpath floor. Drainage on the structure is handled using a slope of the bridge deck and gullies installed on both sides as part of footpaths. The handrail on the bridge structure was made with welded pipe sections. The handrail is 1100mm high.

RC cantilevers are planned for the overhead line poles. There are no overhead line poles on the bridge.

existing cross-section

- Newly designed condition:

The rehabilitation design provides for damp-proofing replacement on the bridge deck, reconstruction of the drainage system and reconstruction of footpaths.

Due to damp-proofing replacement, removal of crushed stone and the the existing damp-proofing cover, traffic closure is planned in the course of such works. Before damp-proofing placement, and if necessary, the bridge deck should be fixed using repair mortar. The new damp-proofing is a spray-type waterproofing.

The existing gullies shall be replaced by new ones. Gullies will be extended up to the bottom edge of the girder and are supported against girder webs in all aspects as per details given in Graphic Documents and the scale with Drainage Design.

Footpaths shall be reconstructed by adding prefabricated cornices, replacing the existing prefabricated slabs with new ones and repairing inner surfaces of the cable ducts. Damp-proofing and duct filling in the footpath is also planned. The filling for the footpath shall be made of sand placed in two layers with cable ducts. New footpath details are given in Graphic documents. The exiting drainage outlets in cable ducts on footpaths shall be closed and new drain tubes installed as per new layout in all things according to the details given in Graphic documents. According to static calculation, pendulums on piers S2 and S3 do not have adequate bearing capacity and should be replaced with new bearings.

In addition, the footpath handrail and expansions joints should also be replaced.

newly designed cross-section

- Superficial damage repair

Superficial damage repair on outer girder surfaces in the form of crack repair by injection and repair of damaged covers. Following superficial damage repair works, it is necessary to protect all the outer girder surfaces with protective coatings.

The planned intervention areas for superficial damage repair are shown in Graphic documents.

- Interventions on the bridge during exploitation

According to the existing documents obtained from ŽICG, we established that no changes occurred in the course of bridge exploitation.

### Static calculation - summary

- .........

### Technology of work execution and rehabilitation works

The construction site provides:

- storage space for the tools and equipment;

- storage space for the material;

- enclosed and secured space for combustible materials and fuels;

- office space for the Contractor and the Engineer;

- compounds for the workers;

- area for the machines;

- other.

Given that each structure has different conditions for the transport of personnel, equipment, machinery and materials, the following principles were adopted:

- The construction site is formed in the area of the structure in case:

- there is an asphalt road or a good macadam road;

- there is free space for construction site capacities.

- The construction site is formed in the area of the nearest railway station and in that case the following applies:

- transport of personnel and material to the bridge is carried out using railway rolling stock during the whole time of works as per established procedures and possible time slots;

- providing a building for the security personnel, a building for workers' accommodation and the tool warehouse at the bridge location.

The following was adopted for bridge M38:

- there is no access road to the bridge

- central construction site should be formed at Mojkovac station at km 321+400 to which there is a road used by trucks and other vehicles

- transport of personnel, equipment, machinery and material from Mojkovac station to the bridge is carried out using railway rolling stock

- personnel can be transported to the bridge by train from Mojkovac to Trebaljevo junction at km 331+100.

- Superstructure

The items of works for which technology of execution is described in detail are as follows:

  - Placement of new damp-proofing under the track prism

Removal of a cover (fine-grained concrete) and removal of existing damp-proofing, sandblasting of concrete surfacing, applying repair mortar, producing new spray damp-proofing with a cover made of prefabricated sheets of extruded polystyrene.

  - Placement of new gullies

Disassembly of existing gullies, treatment of the area with repair mortar, applying damp-proofing and installation of new gullies.

  - Placement of new bridge expansion joints

Disassembly of existing expansions, treatment of concrete surfaces, fitting new expansion joints.

  - Repairing cable ducts

Removing sand from the ducts, local dislocation of SS and TT cables, sandblasting of concrete surfaces, applying repair mortar, installation of drain tubes and applying spray damp-proofing.

  - Repair of cornices on footpaths

Machine removal of the top and lateral surface of concrete cornices, sandblasting the concrete and rebar, producing new section of a cornice with a drip cap.

  - Placement of new handrail on footpaths

Removal of existing handrail on footpaths, placement of new handrail onto previously repaired cornices.

It is essential execute new damp-proofing, new expansion joints and new gullies in a high-quality manner so as to eliminate harmful effect of water on the structure long term. Placement of damp-proofing and the repair mortar coating must be done adhering to the temperature, humidity and hardening times. It is also important to avoid frequent resumption of works when damp-proofing as much as possible.

The above-mentioned items of works are executed under special railway traffic management schemes as follows:

- items 1 and 2 in bridges with gullies at the structure axis are executed under traffic closure and voltage shut-down for a maximum period of time;

- items 2 and 3 are executed under “railway closure for construction works” scheme in the day-time slots (e.g. 1130 to 1630 ) with or without power shutdown;

- items 4, 5 and 6 are executed under low-speed traffic scheme (30 km/h) without power shutdown.

Works defined under items 2 through 6 may be executed under railway traffic scheme a) or b) if this does not interfere with the works defined under item 1 in the Description of works.

- Track works (the bridge and the bridge area)

For the execution of works under items 1, 2 and 3, it is necessary to remove the rails, sleepers and crushed stone ballast all the way to the damp-proofing layer.

Here is the description of items of track works that need to be executed.

- Works to be executed before traffic closure:

- assembly and disassembly of rail anchors in the track area after abutments;

- assembly and disassembly of sleeper anchors in the track area after abutments;

- cutting rails in the embankment at 10m from abutments and at every 22.5m on the bridge with rail drilling and fitting of rail connectors.

2)	Works executed under traffic closure and power shutdown:

- track disassembly (rails, check rails, sleepers, ballast prism) in a field of 15 m;

- assembling the track using existing rails, the new crushed stone ballast, new wooden sleepers and new rail fasteners.

3)	Works executed after construction works in the track area:

- welding rails as CWR, placement of check rails;

- regulating the track by direction and the level line according to current condition elements.

- Works on bearings replacement

Works on piers S2 and S3

- erecting the necessary scaffolding for the replacement of bearings

- removing the existing mask wall;

- installing the auxiliary steel structure, i.e. temporary steel supports for the presses - bearing capacity of min 200t and temporary NAL bearings;

- disassembling the existing pendulum after the structure has been placed onto temporary supports;

- drilling the anchors into the existing pier and laying newly designed dressed stones in cement;

- placing new NAL bearings into their designed place.

    - The list of templates used for the creation of technical documentation

The design engineer used the following documents for the creation of the rehabilitation project:

- The original design - Main design of viaduct at km 47+140 of railway line Gostun - Titograd (developed by Institute for designing ZJŽ Belgrade, 1966.);

- Bridge condition report: Bridge in km. 330+649.90 of railway line Vrbnica - Bar (developed by Pro-inženjering, 2015.);

- Geotechnical elaborate (2018);

- Geodetic survey (2018)

    - List of applied regulations

LAWS AND RULEBOOKS REGARDING THE CONTENTS OF THE TECHNICAL DOCUMENTATION

- Railway law. Official Gazette of RMN, N° 27/2013;

- Law on safety, organization and efficiency of rail transport of Montenegro of 27/12/2013, in force since January 2014;

- Law on spatial development and construction of structures. Official Gazette of RMN, N°51/08, 40/10, 34/11, 47/11, 35713, 39/13;

- Law on construction products N° 18/2014;

- Law on geological researches. Official Gazette of RMN, N° 28/93, 27/94, 42/94, 26/07;

- Law on occupational safety. Official Gazette of RMN, N° 34/2014;

- The Rulebook on content and production of technical documentation - Official Gazette of RMN, N° 23/14, 32/15, 75/15;

RULEBOOKS FOR THE DESIGN

- (316) Rulebook on technical standards for determining the size of the load and categorization of railway bridges, culverts and other structures on railway lines. Edition 1992

EN STANDARDS

- MEST EN 1990:2013-Eurocode - Basis of structural design.

- MEST EN 1990:2013/NA:2013- Eurocode - Basis of structural design - National Annex.

- MEST EN 1991-1-1:2017/NA:2017- Eurocode 1: Actions on structures - Part 1-1: General actions - Densities, self-weight, imposed loads for buildings - National Annex

- MEST EN 1991-1-3:2017-Eurocode 1 - Actions on structures - Part 1-3: General actions - Snow loads.

- MEST EN 1991-1-3:2017/NA:2017- Eurocode 1 - Actions on structures - Part 1-3: General actions - Snow loads - National Annex

- MEST EN 1991-1-4:2016-Eurocode 1: Actions on structures - Part 1-4: General actions - Wind actions.

- MEST EN 1991-1-4:2016/NA:2016- Eurocode 1: Actions on structures - Part 1-4: General actions - Wind actions - National Annex.

- MEST EN 1991-1-5:2017/NA:2017- Eurocode 1: Actions on structures - Part 1-5: General actions - Thermal actions - National Annex

- MEST EN 1992-1-1:2017/NA:2017 -  Eurocode 2: Design of concrete structures - Part 1-1: General rules and rules for buildings - National Annex

- MEST EN 1997-1:2017- Eurocode 7: Geotechnical design - Part 1: General rules - National Annex;

- MEST EN 1997-1:2017- Geotechnical design - Part 1: General rules;

- MEST EN 1998-1:2015 - Eurocode 8: Design of structures for earthquake resistance - Part 1: General rules, seismic actions and rules for buildings.

- MEST EN 1998-1:2015/NA:2015 - Eurocode 8: Design of structures for earthquake resistance - Part 1: General rules, seismic actions and rules for buildings - National Annex

- SRPS EN 1991-1-5:2012 - Eurocode 1: Actions on structures - Part 1-5

- SRPS EN 1991-1-1:2012 - Eurocode 1: Actions on structures -Part 1-1

- SRPS EN 1991-1-3:2012 - Eurocode 1: Actions on structures -Part 1-3

- SRPS EN 1992-1-1:2015 - Eurocode 2: Design of concrete structures: Part 1-1

- SRPS EN 1997-1:2004 - Eurocode 7: Geotechnical design - Part 1

- HRN EN 1991-1-7:2012 - Eurocode 1: Actions on structures - Part 1-7

- HRN EN 1991-1-7:2012/Cor.1:2015 - Eurocode 1: Action on structures -Part 1-7

- HRN EN 1991-1-7:2012/A1:2015 - Eurocode 1: Action on structures -Part 1-7

- HRN EN 1991-2:2012 - Eurocode 1: Action on structures -Part 2

- HRN EN 1992-2:2013 - Eurocode 2: Design of concrete structures -Part 2

- HRN EN 1998-2:2011 - Eurocode 8: Design of structures for earthquake resistance - Part 2

- HRN EN 1998-3:2011 - Eurokod 8: Design of structures for earthquake resistance -Part 3

- HRN EN 1998-3:2011/Cor.1:2014 - Eurokod 8: Design of structures for earthquake resistance -Part 3

- HRN EN 1998-5:2011 - Eurokod 8: Design of structures for earthquake resistace - Part 5

## 2.2.  Technical description - the superstructure

2.2.1	GENERAL

Within the Main Design for the rehabilitation of bridge no. 38, works on the track superstructure in the function of bridge rehabilitation were planned. Technical documents for bridge no. deal with track section 55.00m long, which includes the total bridge length and 10.00m before and after the bridge.

Based on the Conclusion of the Inspection Report on the bridge 38 superstructure condition, and establishing the current condition in the drafting period of the relevant technical documents, works to be done on the bridge track superstructure were defined. The track on the bridge is straight, in a vertical curve; it was produced as a CWR and has check rails.

Measures were adopted to be taken on the bridge track superstructure with the drafting of Priced BoQ, Technical Specifications, Static Stress Check of the Superstructure and the Arrangement of the Railway Tracks on the Bridge.

Total value of superstructure works for the bridge no. 38 amounts to 44,795.22 €.

2.2.2	REQUIRED MEASURES AND SEQUENCE OF WORKS ON SUPERSTRUCTURE WHEN EXECUTING BRIDGE REHABILITATION WORKS

For the execution of rehabilitation works (placing new damp-proofing under the ballast prism, installation of new gullies and expansion joints) the following works on the superstructure were planned, where the bridge length and the length on the embankment of 10 m from abutments, before and after the bridge are taken into account:

- arresting CWR before and after the bridge with the installation of rail anchors;

- disassembly of the existing track in segments up to 22.5m with the track and checkrail removal, whereby it is necessary to cut the existing track and drill rails for the connections at the joint, perform any disassembly of fasteners and sleepers as per technology of track and check rail removal, as well as disassembly of rail anchors on the bridge;

- removal of the current crushed stone ballast all the way to the damp-proofing protective layer;

- installation of the track and check rails in segments up to 22.5m with new crushed stone, new sleepers and new fasteners, with a fitting of connectors and the necessary fasteners for joining running rails and check rails;

- regulating the track by direction and the level line as per elements of current condition;

- disassembly of rail joints on running rails before welding;

- welding the track as CWR;

- disassembly of rail anchors, installed for the purpose of arresting CWR - 74 rail anchors before the bridge and 168 after the bridge (installed in the length of 25.16m before the end switch of Trebaljevo station, 4 rail anchors for each sleeper) and installation of rail anchors on and after the bridge (before the beginning of the entrance point of Trebaljevo station) in accordance with the existing number and position of rail anchors;

- final track regulation.

Bridge superstructure works for bridge rehabilitation, particularly the disassembly and the assembly of the existing track, are executed under traffic closure while low-speed traffic scheme should be employed on the bridge section during rehabilitation works.

2.2.3	STATIC CALCULATION - SUMMARY

The basic elements of the superstructure used in the calculation of track stresses and stability are as follows:

- 49 E1 rails, grade R260 (900A)

- wooden sleepers L=260cm, at axial spacing of 60cm

- "K"-type fasteners.

The RC structure is a continuous beam on three spans of L=10.00+15.00+10.00=35.00m, in a straight line. The bridge extends over a dry ravine and the terrain is a steep cut. The cross section of the spanning structure consists of two main RC girders with a bridge deck between them and cantilevered overhangs on both sides. Cross girders are constructed above supports, in the middle of end spans and in the thirds of the middle span. Fixed supports are on the abutment S4 (towards Bar), and the rest of the piers are suported on RC pendulums. The track on the bridge follows the alignment, in a vertical curve, welded as a CWR.

Having in mind the regulations from the Rulebook on the maintenance of the railway tracks superstructure (“Official Gazette of Montenegro, no. 42/2016“ published on 11/7/2016), as well as the valid EN standards and the UIC Announcements, and in addition to the stance that there is no relative displacement between the bridge and the LRT (“Study of various methods adopted by world railways to continue LWR over bridges“) the calculations and analyses are done for the temperatural change impacts on the rails without any impact of the bridges on the rail (just like on open tracks, that is, as if the tracks were practically observed on the embankment).

Characteristic results

a) For the LRT of 49E1 rails, the maximum compressive force is:

N= -661.50kN

b) For the LRT of 49E1 rails, the maximum tensile force is:

N= 817.14kN

c) The inspection of track stability against buckling in extreme summer temperatures employing the Mischenko energy model was conducted along the alignment on the bridge .

The calculated safety coefficient was higher than 1.20, so the track is deemed to be stable with a well tamped ballast.

d) Inspection of crack size on rail cracking in winter was made, and it was determined that, on rail cracking, the crack size will be 2.52cm<10cm, i.e. traffic safety shall not be at risk in case of rail cracking.

2.2.4	TRACK DEVELOPMENT

Based on the performed analysis and the obtained calculation results, the track should be developed in the following way meeting the following requirements:

- The substructure before and after the bridge must be completely and appropriately executed (cross-section, drainage, stabilization)

- Crushed stone ballast before, after and on the bridge must be uncontaminated, having the adequate quality and grain size distribution, with a ballast prism intended for CWR. On sections where rail anchors are placed, additional tamping of ballast prism should be performed.

- Before welding, the track (on the bridge and outside the bridge) must be completely regulated by direction and the level line as per the elements of the current condition.

- The calculation was made under the condition that both the railway line and the track on the bridge are welded as CWR.

- The required temperature for CWR tempering is +230C ± 30C

- Before arresting CWR, 74 rail anchors are to be installed on approximately 50m of track before the bridge, and another 168 rail anchors on 25.16m, before the beginning of the entrance point of Trebaljevo station, (4 rail anchors for each sleeper). The rail anchors installed before the bridge are to be disassembled after the final welding. Rail anchors on and after the bridgeshall be installed in accordance with the existing number and position of rail anchors; with the disassembly of surplus rail anchors which were installed in order to arrest CWR, but only after the final welding.

- Installation of devices against lateral track movement is not necessary, that is, the track in a straight line with a well tamped ballast provides the stability against track buckling.

- Permanent labels for monitoring CWR longitudinal and transversal displacement (in the bridge area) should be put in the following places:

- at the first sleeper after the bridge

- at the first sleeper before the bridge.

Labels are buried at both sides of the track on stable soil.

Labels are placed before completing CWR formation, and are calibrated immediately after CWR tempering in the presence of the Supervisor.

Chainage labels can be used as permanent labels for CWR monitoring, provided they are in the immediate vicinity (within 3 m) of the places where labels are placed. Permanent labels are not included in the Priced Bill of Quantities.

- As check rails, the exiting rails already laid on the bridge shall be used, which are interconnected at its end with an oak key, whereby the new fasteners are installed on the sleeper connection with the running rails and check rails, which connection is made at each sleeper, in addition to new fasteners for bridge ends and new additional fasteners for check rail joints.

- All the necessary track maintenance works should be done on time, in a quality manner and within the allowed temperature ranges. It is also necessary to ensure enhanced supervision on the section at extreme rail temperatures (below  -10°C and over +35C ).