# 2.	TECHNICAL REPORT

________________________________________________

  - Introductory remark

There is a bridge structure after one km of railway 447+435,25. The arcuated construction of the bridge was built with seven spans of axial land piers L=7x14,2m. The structure is situated in residential area of Sutomore and it is completely covered with rectangular chipped stone except the lower part of concrete of arcuated girders. There are traffic roads in spans of S6-S7-S8, and there is a small stream in a concrete canal in spans of S5-S6 and there is a watercourse of streams from the surrounding hills in the spans of S3-S4. The area around the bridge is urbanized and the ground is stable.

Figure  Excerpt from the Report

The sewerage system with vertical gullies was built where the arcuated girders lean.

Figure  The view of the railway track on the bridge

Figure  The side view of the bridge structure

  - Drainage system

Due to inadequately built drainage system, and the lack of the same in some places, there is uncontrollable leakage through the concrete of arcuated girders. As a result, the concrete is significantly affected by calcification and completely soaked in water. There are some vertical gullies in some spans where the arcuated girders lean. The gullies are in a bad condition, corroded and out of order.

Figure 4	Drainage system

Figure 5	Gully