# 2.	TECHNICAL REPORT

________________________________________________

  - Introductory remark

There is a bridge structure after one km of railway 323+076,82. The bridge is a continuous construction with three spans L=13,50+23,00+13,50m, which in its interior span is angling over the motorway Bijelo Polje-Kolašin. The cross section is comprised of AB girders and AB concrete slabs with haunches, also suppoerted with diagonal AB girders (leaning in spans too). Around land piers S2 and S3 the cross section is comprised of lower AB slab too, therefore it is a box girder. The land piers S1, S2 and S3 in the bridge structure are supported with the steel slab while the land pier S4 is supported with AB pendulum.

Figure 	Excerpt from the Report

Figure  The view of the railway track on the bridge

Figure  The side view of the bridge structure

  - Drainage system

- There is a drainage system on the bridge structure. Considering concrete slab camber, the drainage from the railway goes through the vertical gullies on both sides of the bridge structure. Vertical gullies are placed on a proper distance in the middle of a bracket on the sidewalk. Between S2-S3 there is a horizontal gutter below the gully, the purpose of which is to prevent catchment on the motorway. Left horizontal gutter is in a good condition, while the right one is damaged. Vertical gullies are made of steel, diameter is 100mm, and their surface is corroded. Wet concrete and catchment down the bracket and the main girder have been noticed only around one vertical gully. It can be said that the drainage system is in a bad condition.

Figure 4 Drainage system

Figure 5 Gully